How to use?
Slicer model at normal profile in Cura , save version 1

Slicer model again , now change: ( I recommend create a second profile frome current profile)
Layer height = 0.1
Wall line count= 1
Top Layer = 1
Bottom Layer = 0

Infill = 0
Minium Time Layer = 0
Turn off support

Edit file Python, change name files:

F is version 1 at 0.2mm

F2 is version 2 at 0.1mm

F3 = output file

If you use 0.3mm or 0.4mm layer height, change “height=” your value

Done, now run code.

Open output gcode to check layers again.

If nothing wrong, copy to sdcard and print